| VERIficaton report on GHG emissions reduction TEDOM ENERGO - Energy utilization of the landfill gas orlík iv |  |  |  |
| --- | --- | --- | --- |
| KIND OF REPORT |  | final |  |
| Date of issue |  | 28 JANUARY 2011 |  |
| REPORT TITLE |  | Verification Report on GHG Emissions Reduction |  |
|  |  | TEDOM a.s. - Energy utilization of the landfill gas Orlík IV |  |
| ORDER No. |  | 51 638 610 |  |
| CLIENT |  | TEDOM a.s. |  |
| AUTHOR |  | Ing. Jan Kalužík |  |
| Participator |  |  |  |
| Verifier |  |  |  |
| division manager |  | Ing. Michal Fiala |  |

ORGREZ, a.s., Certification Body of GHG emissions,

Hudcova 76, 657 97 Brno, registered at accredited CIA under No. 3198

| ORGREZ, a.s. CertifiCation body of GHG emissions Hudcova 76 657 97 Brno Tel: 541 613 302 Fax: 541 613 299 E-mail: tepchem@orgrez.cz | E-mail: tepchem@orgrez.cz GSM: 724 220 181, 602 595 427 tel.: 541 613 291, fax: 541 613 299 |
| --- | --- |

Table of Contents

1 Corresponding Legislation	4

2 General Information on the Implementor	4

2.1 Name of Project	4

2.2 Address of Plant	4

262 72 Příbram	4

2.3 Operator	4

2.4 Operator‘ s Registered Number	4

2.5 Type of Plant	4

2.6 Number of Resolution for Permission to Release GHG Emissions	4

2.7 Reporting period	5

3 Description of Plant	5

4 Subject and Verification Process	5

4.1 Documentation - Operator	5

4.2 Reliability, Credibility and Accuracy of Data	5

4.2.1 Emission Factors - Selection and Application	5

4.2.2 Calculations of Total Emissions Reduction	6

4.2.3 Usefulness of Selection and Application of Measuring Methods	7

4.2.4 Trend analysis	7

4.3 Verification Process Description	8

4.3.1 Variables Involved in Calculations	8

4.3.2 Assessment of Compliance with Verification Methodology	8

4.3.3 Computer Information Systems	8

4.3.4 Measurement and Measuring Devices	9

4.4 Summary of Modifications	9

4.4.1 Technological Modifications	9

4.4.2 Verification Methodology Modifications	9

4.4.3 Organizational Modifications	9

4.5 Conclusions and Recommendations Resulting from Verification Reports	10

5 Evidence of Verification Data Quality	10

5.1 Requirements for Accuracy	10

5.2 Compliance of Applied Methodology with Current Legislation to Determine Emissions	10

5.3 Missing Documentation	10

5.4 Declaration on Emission Data Quality	10

6 Information on the Total Amount of GHG Emissions	11

7 Client's Declaration	11

8 Date and Signature of Authorized Person on behalf of Certification Body	11

9 Information on Certification Body	12

9.1 Company Name	12

9.2 Company Address	12

9.3 Authorized Person	12

9.4 Authorization	12

9.5 Verification Team	12

9.6 Authorization	12

9.7 Accreditation	12

# Corresponding Legislation

- Methodical instruction issued by the Department of climate changes of the Ministry of Environment due to calculating a reference level (Baseline) for energy-projects landfill gas utilization

- Decree of Ministry of Industry and Trade No. 345/2002 specifying measures for compulsory verification and meters subject to type approval

- Decree No. 65/2006 which modifies the Decree of Ministry of Industry and Trade No. 345/2002 Coll. specifying measures for compulsory verification and meters subject to type approval

- Directive No. 80/2008 Coll., on the National Allocation Plan of the Czech Republic for the period of 2008 - 2012

# General Information on the Implementor

## Name of Project

TEDOM a.s. -  landfill Orlík IV

## Address of Plant

Municipal waste landfill Orlík IV

## Operator

TEDOM a.s.

## Operator‘ s Registered Number

284 66 021

## Type of Plant

Incineration plant combusting landfill gas and generating electricity. The equipment is involved in the Joint Implementation Project and its nominal heat input is below 20 MW.

## Number of Resolution for Permission to Release GHG Emissions

As the plant with its input is not incorporated in the trade system demanding emission permissions (Directive issued on February 25, 2008 on the National Allocation Plan of the Czech Republic specifying the trading period 2008 - 2012, Annex No. 2), no amount of permissions has been allocated to  TEDOM a.s.,  landfill Orlík IV and this source is not listed in NAP for the trading period at all.

## Reporting period

This verification is related to emission announcement of  landfill Orlík IV for 2009 and 2010.

# Description of Plant

Incineration power plant with its total heat input 0,423 MW is operating on the  landfill Orlík IV and consisting of one incineration sources:

  - landfill combustion - cogeneration unit, with heat input rate of 0,423 MW

# Subject and Verification Process

## Documentation - Operator

- Invoices for electricity supply

- Electricity production and own consumption records

- Documents about calibration electric meters

- CO2  emissions reduction data during 2009 - 2010

- Technical parameters of each operating part and their operational process

## Reliability, Credibility and Accuracy of Data

### Emission Factors - Selection and Application

- electricity emission factor 1.11 tCO2/MWh (2008 - 2011) –  this value is used in compliance with Methodical instruction due to calculating a reference level  for energy – projects landfill gas utilization

- heat emission factor 0.202 tCO2/MWh –  this value is used in compliance with Methodical instruction due to calculating a reference level  for energy – projects landfill gas utilization

- CH4 emission factor 1,31 tCO2/MWhP – this value is used in compliance with Methodical instruction due to calculating a reference level  for energy – projects landfill gas utilization

### Calculations of Total Emissions Reduction

Landfill gas combustion produces CO2 emission reductions in one cogeneration units  installed in  landfill Orlík IV. To determine the baseline several levels are involved according to the origin of emission savings. Three fundamental types of emissions can be involved in the project:

- as a substitute for fossil fuel during electricity generation - Ee

- as a substitute for fossil fuel during heat power generation - Et

- by landfill gas dissolution (% share of CH4 is incorporated) escaping from the landfill - ECH4

The total annual CO2eqv. emissions saving reached due to the project implementation will be calculated from partial calculations of emission savings as follows:

E = Ee + Et + ECH4

#### Fossil fuel substitution during electricity generation

Generated electricity is measured with calibration meter on the site of distributing to the distribution network, moreover, consumption inside each unit itself is measured. Factual operational values reached due to the project implementation are incorporated in calculating.

Annual electricity generation in MWh is calculated by subtracting of operational values according to the formula :

Annual electricity distributed into the network (MWh) = gross annual electricity (MWh) – consumption inside the unit (MWh)

Emission saving calculation

To transform the same electricity amount from heat power generated during coal burning we can calculate CO2 amount into the atmosphere:

Annual emissions saving during electricity generation (tCO2) =  annual electricity distributed into the network  (MWh) x electricity emission factor (tCO2/MWh)

#### Fossil fuel substitution during heat production

Values measured with calibration meter during distributing to the heat network are base for calculating the reduction, i.e. operational values reached due to the project implementation.

Emission saving calculation

Within internal assignment of real emission reductions measured values of heat supply with calibration meter are used due to the project verification. Then CO2 emission saving generated due to fossil fuel substitution:

Annual emissions saving during heat production (tCO2) =  utilized annual heat (MWh) / compensatory source efficiency (-) x heat emission factor (tCO2/MWh)

There is no utilization for the produced heat in landfill Orlík IV. Therefore total heat output of the CHP unit is led into air. The total emission reduction doesn´t cover emission reduction from heat utilization.

#### Landfill gas dissolution escaping from the landfill

The amount of methane generated by the landfill is calculated from electricity production. Methane is the only one which can utilize landfill gas as energy, i.e. it is possible to do precise calculations of its consumption in cogeneration unit with familiar efficiency.

Fuel input necessary for annual output calculated from real electricity production:

Fuel input (MWh) =  gross annual electricity (MWh) / electrical efficiency of cogeneration unit (-)

Emission saving resulting from landfill gas utilization is calculated consequently:

Annual emission saving due to landfill gas incineration (tCO2) = fuel input (MWh) x CH4 emission factor (tCO2/MWh)

Fuel input necessary to maintain annual output is not measured directly but it is calculated from the measured value of electricity production and electricity production efficiency in each unit. Efficiency of cogeneration unit to be used for calculating fuel input is as follows :

- CHP TEDOM Cento T150 SP BIO CON		33,6%

Calculation is performed on the basis of the Methodical instruction due to calculating a reference level for energy-projects landfill gas utilization

### Usefulness of Selection and Application of Measuring Methods

Selected Method of  CO2 emission reduction verification and assignment was based on the requirements of valid legislation.

### Trend analysis

Trend analysis determines CO2 emissions development in the period of 2005 - 2009 in accordance with production parameters , particularly, in compliance with the electricity amount generated in cogeneration units.

| Year | 2009 | 2010 |
| --- | --- | --- |
| CO2 emissions [ t] | 218 | 935 |
| E production [MWh/year ] | 45 | 192 |
| Correlative coefficient | 0,2058 | 0,2049 |

As in the aforementioned annual CO2 emission reduction ratio remains to generated electricity approximately stable in the relevant period.

## Verification Process Description

### Variables Involved in Calculations

Process analysis was launched by the crosscheck of variables involved in calculations. Validity was verified consequently:

- gross amount of electricity production in the period of 2009 - 2010

- amount of electricity distributed into the network in the period of 2009 - 2010

- application and use of  regular electricity emission factor

- application and use of  regular heat emission factor

- application and use of  regular CH4 emission factor

Identified disagreements:

Emission reductions achieved by dissolution of landfill gas escaping from the landfill as well as the applied method of assignment, as for the method and applied electricity efficiency of cogeneration unit when determing fuel input, are in compliance with the Methodical instruction. However, to determine fuel input more precise method can be even considered, i.e. the method based on flow measurement of landfill gas. To determine this it is essential to know the landfill gas heating value .

Conclusion of this verification part

All values of emission factors have been applied in accordance with the Methodical instruction due to calculating a reference level  for energy-projects landfill gas utilization.

### Assessment of Compliance with Verification Methodology

Process analysis was furthermore dealt with the compliance of the applied method of  CO2 emission reduction verification and assignment resulting from the project in  landfill Orlík IV in accordance with the requirements of valid legislation. The Verifier has checked and acknowledged the validity of the assigned emission reductions by means of crosscheck calculation.

Identified disagreements:

None have been found in correspondence with valid legislation.

Conclusion of this verification part:

No specific conclusions resulting from this verification part.

### Computer Information Systems

To calculate CO2 emission reductions MS Excel file is being used which can process annual balance sheet. It has been verified:

- validity of data entry

Identified disagreements:

None have been found in correspondence with valid legislation.

Conclusion of this verification part:

No specific conclusions resulting from this verification part.

### Measurement and Measuring Devices

Subsequent measuring devices have been verified in the process analysis:

- electric meter of power supply into the network

- electric meters of electricity produced in each cogeneration unit

It has been verified that these measuring devices:

  - have been approved by the relevant body

  - comply with the requirements in the Metrology Act, are provided with AMS and ČMI official brands (seals are intact), and are liable to regular validation in accordance with the Decree of  MIT No.345/2002 Coll.

Identified disagreements:

Measuring devices (electric meters) which measure gross electricity generation at each cogeneration unit do not correspond with requirements stated in the Metrology Act, and they are not provided with official brands and seals (are neither calibrated nor validated regularly).

Conclusion of this verification part:

It is compulsory to calibrate this measuring device to verify the accuracy of device applied to determine gross electricity production.

## Summary of Modifications

### Technological Modifications

In 2009 one fundamental modification in applied technology was performed in the Orlík IV landfill:

- In the 2009 was put out of operation the CHP unit TEDOM Cento T150 SP BIO CON

### Verification Methodology Modifications

No modifications have been specified in the methodology of CO2 emission reduction verification and assignment.

### Organizational Modifications

In August 2009 the owner and operator of the CHP unit was changed from Technické služby Děčín to TEDOM s.r.o.  From November 2010 the TEDOM ENERGO s.r.o. was changed to TEDOM a.s..

## Conclusions and Recommendations Resulting from Verification Reports

Considering results arising from verification work  it is recommended to monitor, identify and assign  CO2 emission reduction in  landfill Orlík IV in a following way:

1. Values of all used emission factors have been applied in accordance with the Methodical instruction due to calculating a reference level  for energy-projects landfill gas utilization.

2. Electric meters measuring gross electricity generated in each CHP are not correspondent with the requirements determined in the Metrology Act as for calibrating and regular validation. I recommend to perform calibrating of individual measuring device and their subsequent validation or alteration for calibration measures.

# Evidence of Verification Data Quality

## Requirements for Accuracy

The operator followed the requirements in accordance with the Methodical instruction due to calculating a reference level  for energy-projects landfill gas utilization during CO2 emission reduction assignment in  landfill Orlík IV. No disagreement which would affect the quality and reliability of the assigned CO2 emission reduction has been identified.

## Compliance of Applied Methodology with Current Legislation to Determine Emissions

Assigned methodology of CO2 emission calculations in the landfill Orlík IV to determine CO2 emission reductions for 2009 and 2010 is in compliance with the approved Methodical instruction  due to calculating a reference level  for energy-projects landfill gas utilization.

## Missing Documentation

During verification work the Certificate body did not come across any missing documentation which the operator would not be able to present.

## Declaration on Emission Data Quality

The verifier has been satisfied sufficiently due to crosscheck of the sufficient amount of evidence because assignment on GHG emission reductions from landfill Orlík IV do not involve any relevant disagreements or faults.

# Information on the Total Amount of GHG Emissions

| CO2 Emission | 2009 | 2010 |
| --- | --- | --- |
| AAU | 43 | 188 |
| ERU | 175 | 747 |

| Project | Vintage year | Total CO2 emission |
| --- | --- | --- |
| Orlík IV landfill | AAU up to 2007 | 0 t |
|  | AAU 2008- 2010 | 231 t |
|  | ERU 2008- 2010 | 921 t |
|  | Total | 1 152 t |

# Client's Declaration

We confirm that we have provided the Certification body with all necessary data and information and, consequently, we will make sure that the Validation Report or any of their part shall not be misused in any invalid way.

In……………on……………			                          …………………………………………

signature of authorized representative

# Date and Signature of Authorized Person on behalf of Certification Body

Brno, 28 January 2011			            		Brno, 28 January 2011

……………………………….			……………….……………………………

signature of Head Auditor				signature of Head of the Certification Body

# Information on Certification Body

## Company Name

ORGREZ, a.s.

## Company Address

Hudcova 76

657 97 Brno

## Authorized Person

Ing. Jan Kalužík

## Authorization

MoE No. A-01-10/568 from 12 January 2010.

## Verification Team

Name					position

Ing. Jan Kalužík			auditor

## Authorization

## Accreditation